# Schematic review for EG800Q-EU application

Team: ANZ

Reporter: Xianghao Kong

Date: 25th Mar, 2026

Solar charging Circuit:

- Cathode of D4 should be connected to GND.

- C26, C27 should switch position. Smaller capacitance to be closer to circuit output.

- C23 is drawn is on the left side of the SCH. However, this is a very important decoupling capacitor and needs to be placed very close to the VCC pin of U5. It is also recommended to add 1uf and 0.1uf as decoupling capacitors as well for high frequency filtering.

- PCB layout for this part is very critical as it contains power switching circuit. Please follow regular buck converter layout guideline for this part.

Power Architecture:

- Buck converter circuit from +BATT to B3V3 is drawn incorrectly and will not work, more specifically, the feedback is connected incorrectly. Please refer to the datasheet of U4 for correction. Also please note the component placement and PCB layout is also very critical for this buck converter.

- 1uf and 0.1uf is also highly recommended to be placed in parallel with C4 as decoupling capacitor.

- C7, C8 120pf is not big enough as output capacitor and will cause significant output voltage ripple. Consider adding 1uf, 10uf together in parallel.

- If B3V3 is 3.3V output, then it is not enough to power EG800Q-EU. It is recommended to use 3.8V as module’s main power source.

- LDO circuit U3 from 12V drop to 3V3 is too aggressive. Large voltage drop will lead to potential overheat. It is recommended to use the output of buck circuit(3.8V) as the input of this LDO.

GPS Power Gate Circuit:

- Please ensure that Q1 and Q2 are placed correctly. Based on my observation, the drain and source appear to be reversed. Could this be a schematic symbol issue?

- Pull down or pull up resistor is recommended on GPS_PWR line based on your application to define a initial state for this line before ESP32 boots up.

MAX-M10 circuit:

- At least one decoupling capacitor 0.1uf is needed next to the VCC pin.

- V_IO is connected to anything and will cause uart to fail. Refer to datasheet for detail.

- It is recommended to add ESD protection for RF trace. Please note, you can only use ultra-low capacitance (0.1pf to 0.5pf) ESD chip on rf  line.

USB circuitry:

- At least one decoupling capacitor 0.1uf is needed next to the VDD pin of U6.

LTE circuit:

- The PWRKEY control cannot be implemented through the level-shifting chip U8. During power-up, VDD_EXT is at 0V, which causes the OE pin of U8 to be pulled low, thereby disabling U8. As a result, U8 is non-functional at this stage, and the PWRKEY pin cannot be driven low to power on the EG800Q-EU.

- It is recommended to design the PWRKEY control with open drain circuit. Please refer to EG800-EU hardware design manual. This prevents any potential voltage mismatch issue.

- R25 is suggested to be deleted.

- Ultra-low capacitance ESD protection diode is also recommended on the rf transmission line.

- Test points are recommended on USB_VBUS, USB_DM, USB_DP to allow quick FW upgrade or future debugging purpose.

- Test points are also recommended on debug uart channel for debugging purpose.

SIM holder:

- It is recommended to replace the series resistor R14, R15, and R16 with 0ohm resistors.

- EG800-EU support SIM hot swap, and you chosen card holder also seem to support this function. You can try to implement this in the circuit.

Other general advice:

- Test points are recommended on all important nodes to allow easier debugging.

- 0ohm resistors are recommended between different power stages so that easier power isolation can be achieved when needed in debugging.

- 4-layer stack up board is recommended for boards with multiple power switching circuit.